PROOV

Provenance-Verified Emissions Monitoring using Cross-Modal Sensor Fingerprinting and Tamper-Evident Logging

Hardware Schematic Verification Summary  |  BACSE324 — Embedded Systems Architecture  |  VIT Vellore

# 1. Purpose of This Document

This document records the final, verified hardware architecture for the PROOV two-node prototype and summarizes the iterative design-review process used to reach it. The schematic went through multiple AI-assisted generation rounds; each round was independently checked against the project's architecture specification, and several rounds surfaced real wiring or electrical-safety errors that were corrected before the design was accepted. This log is included as evidence of a rigorous verification process, suitable for the course report, the GitHub repository, and any prior-art / patent documentation.

# 2. Final Verified Architecture — Summary

The accepted design implements the three-layer trust chain on two independent power domains:

- Heater supply (5V, always-on, unswitched) — powers the MQ gas sensor's heater element continuously, with no relay or GPIO control in its path.

- Relay/equipment supply (5V, GPIO4-switched) — powers the relay, the external test equipment (fan/motor/lamp), and the INA219 current sensor, which sits in series in the equipment's return leg.

- ESP32 logic domain (3.3V) — powers the ESP32, DS3231 RTC, SD card module, and the INA219/DS3231 I2C signal lines, with pull-up resistors referenced to 3.3V only.

This separation is what makes Layer 1 (physical plausibility) meaningful: the gas reading and the equipment's power draw are two independent physical signals, measured on two independent circuits, that are only compared in software.

## 2.1 GPIO Pin Assignments (Final)

| GPIO | Function | Bus / Type | Notes |
| --- | --- | --- | --- |
| GPIO21 | I2C SDA | I2C (shared) | INA219 (0x40) and DS3231 (0x68) |
| GPIO22 | I2C SCL | I2C (shared) | INA219 (0x40) and DS3231 (0x68) |
| GPIO23 | SPI MOSI | SPI | SD card module |
| GPIO19 | SPI MISO | SPI | SD card module |
| GPIO18 | SPI SCK | SPI | SD card module |
| GPIO5 | SPI CS | SPI | Idles high via pull-up; safe strapping-pin use |
| GPIO34 | Analog input (ADC1_CH6) | Analog, input-only | MQ gas sensor AO |
| GPIO4 | Digital output | GPIO | Relay control (switches external equipment only) |

GPIO0, GPIO2, and GPIO12 are deliberately excluded from the design — these are ESP32 boot-strapping pins, and external hardware holding them in the wrong state at power-on can cause boot failure or unintended flash-mode entry.

## 2.2 Power Domain Reference

| Domain | Voltage | Powers | Switching |
| --- | --- | --- | --- |
| Heater supply | 5V | MQ sensor heater element | None — always on |
| Relay / equipment supply | 5V | Relay module, external equipment, INA219 (sense path) | GPIO4 via relay |
| ESP32 logic rail | 3.3V | ESP32, DS3231, SD card, I2C pull-ups, INA219/DS3231 logic supply | N/A — continuous |

# 3. Design Review Log

The table below summarizes the issues identified and corrected across the review process. Each was caught before being accepted into the final design.

| Round | Issue Identified | Risk | Resolution |
| --- | --- | --- | --- |
| 1 | INA219 wired across the MQ heater instead of the external equipment | Defeats Layer 1 — no independent second signal to correlate against | Rewired INA219 in series with external equipment only |
| 2 | Relay wiring reused GPIO pins inconsistently between diagram and table | Ambiguous build reference | Standardized on one wiring topology across all outputs |
| 3 | Pin reassignment introduced GPIO12 (SD MISO) and GPIO2 (relay) | ESP32 boot-strapping pins — risk of boot failure / brick | Reverted to safe pin set: 21, 22, 23, 19, 18, 5, 34, 4 |
| 4 | Relay line shown driving the MQ heater directly | Breaks heater/equipment isolation, re-couples Layer 1 signals | Decoupled heater onto its own always-on, unswitched 5V supply |
| 5 | I2C pull-up resistors referenced to 5V; relay spec implied GPIO sourcing full coil current | 5V on ESP32 GPIOs risks pin damage (not 5V-tolerant); GPIO cannot source 70mA | Pull-ups corrected to 3.3V; relay module confirmed to use onboard transistor driver |
| 6 | Final self-audited version cross-checked against full specification | N/A — verification pass | All checks passed; design accepted as buildable |

# 4. Final Verification Checklist

The accepted design was checked against the following criteria before approval:

✓ INA219 is in series with the external equipment, not the MQ heater

✓ MQ heater runs on an independent, always-on 5V supply with no relay/GPIO in its path

✓ GPIO0, GPIO2, and GPIO12 are not used anywhere in the design

✓ All eight required GPIOs (21, 22, 23, 19, 18, 5, 34, 4) are assigned uniquely, with no conflicts

✓ I2C pull-up resistors reference the 3.3V rail only

✓ Relay coil current is sourced from the 5V rail via the module's onboard transistor driver, not directly from GPIO4

✓ Layer 1 logic correlates MQ gas reading with INA219 equipment power draw

✓ Layer 2 logic samples the MQ heater's resistance curve independently of the INA219

✓ Layer 3 logic hashes (previous_hash + timestamp + gas_value + INA219_power + fingerprint_result) and verifies against the SD card's stored chain

# 5. Final Circuit Diagram

# 6. Conclusion

The PROOV two-node hardware architecture has been verified across six rounds of AI-assisted schematic generation and manual review. Every identified wiring hazard — miswired current-sense placement, unsafe GPIO strapping-pin use, cross-coupled heater/relay control, and an out-of-tolerance I2C voltage domain — was caught and corrected prior to acceptance. The final design isolates the two physical signals that Layer 1 depends on (gas concentration and equipment power draw), keeps the ESP32's logic pins within their safe operating domain, and avoids all boot-critical strapping pins.

This design is considered ready for breadboard assembly. Recommended next steps: begin the MQ sensor 24–48 hour burn-in period immediately, proceed with breadboard assembly per Section 2, and produce a formal KiCad schematic (with an electrical-rules check) for the final course submission and GitHub repository, using this document as the verified reference design.